BET 22 Lesson 5 Rewinding and Repair of Split-Phase and Capacitor Type Motors

SINGLE-PHASE ELECTRIC MOTOR REWINDING

| MOTOR TYPE: | WASH MOTOR SPECIAL DUTY REVERSIBLE CAPACITOR RUN MOTOR |  |
| --- | --- | --- |
| MODEL/ SERIAL NUMBER: |  |  |

NAMEPLATE DATA

| VOLTS: | 220 v | AMPS: | 1.1 | RPM: | 255rpm | CAP (µF): | 10 µF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PHASE | 1 | S.F.: |  | H.P. (W) : | 200 w | Hz: | 60 hz |

WINDING DATA

| SLOTS: | 24 | POLES: | 4 |
| --- | --- | --- | --- |
| ENDROOM (cm) |  | INSULATION THICKNESS (mils) |  |
| Front | Slot Liner | Slot | Cover |
| 2.0 cm | 2.3 cm | 0.109 | 0.125 |
| RUNNING WINDING |  | STARTING WINDING |  |
| COIL SPAN/PITCH | TURNS | COIL SPAN/PITCH | TURNS |
| 1-6 | 250 | 1-6 | 250 |
| 1-4 | 160 | 1-4 | 160 |
| WIRE SIZE | 28 | WIRE SIZE | 28 |
| WEIGHT | 500 g | WEIGHT | 500 g |

COIL PLACEMENT DATA

MOTOR CONNECTION DIAGRAM

COIL CONNECTION SCHEMATIC DIAGRAM

BET 22 Lesson 5 Rewinding and Repair of Split-Phase and Capacitor Type Motors

SINGLE-PHASE ELECTRIC MOTOR REWINDING

Single-Phase Electric Motor Rewinding Activity Rubric

| Criteria | Total Points | Excellent | Very Good | Satisfactory | Needs Improvement |
| --- | --- | --- | --- | --- | --- |
| - Data Gathering and Initial Inspection | 4 | 5 | 4 | 3 | 2 |
|  | 5 pts | Complete, accurate, well-analyzed data | Mostly accurate with minor omissions | Adequate data, some analysis flaws | Incomplete or unclear |
| - Stripping and Cleaning | 4 | 5 | 4 | 3 | 2 |
|  | 5 pts | Clean removal, no damage | Slight residue, no damage | Mostly clean, minor flaws | Sloppy or incomplete |
| - Slot Liner Formation | 3 | 5 | 4 | 3 | 2 |
|  | 5 pts | Precise and snug fit | Slight gaps or wrinkles | Mostly consistent | Loose or misaligned |
| - Coil Insertion and Insulation | 6 | 8 | 6 | 5 | 3 |
|  | 8 pts | Properly placed, well insulated | Minor insulation flaws | Functional, some misalignment | Inconsistent or loose |
| - Termination and Connection | 6 | 7 | 6 | 5 | 3 |
|  | 7 pts | Correct, tight, neat, labeled | Slight labeling or routing flaws | Acceptable with minor errors | Loose or unclear |
| - Coil Tying and Slot Securing | 4 | 5 | 4 | 3 | 2 |
|  | 5 pts | Uniform, tight, secure | Slight unevenness | Generally secure | Inconsistent or loose |
| - Assembly and Final Testing | 8 | 10 | 8 | 6 | 4 |
|  | 10 pts | Fully functional, neat, all tests passed | Minor test issues, clean build | Operates with flaws | Partially functional |
| - Safety Procedures | 3 | 5 | 4 | 3 | 2 |
|  | 5 pts | All PPE, safe practices, tidy area | Minor lapses in PPE or safety | Basic PPE and awareness | Inconsistent safety use |
| TOTAL | 38 |
|  | 50 |

BET 22 Lesson 5 Rewinding and Repair of Split-Phase and Capacitor Type Motors

SINGLE-PHASE ELECTRIC MOTOR REWINDING

Individual Task Participation Rubric

| Criteria | Score | Excellent (2.5 pts) | Good (2 pts) | Satisfactory (1.5 pts) | Needs Improvement (1 pt) |
| --- | --- | --- | --- | --- | --- |
| - Initiative and Task Ownership Voluntarily starts assigned work - Prepares tools and materials - Organizes own workflow - Takes responsibility for tasks | 1.5 | Always initiates and completes tasks independently and on time | Often starts tasks with minimal prompting | Sometimes needs guidance and reminders | Rarely initiates or avoids responsibility |
| - Technical Involvement Winding coils - Stripping and cleaning wires - Wiring and connections - Assembly and mechanical fitting - Testing with measuring tools | 1.5 | Performs all technical tasks with high skill, accuracy, and safety | Completes most technical tasks with minor errors | Contributes with assistance; moderate errors observed | Limited participation; lacks skill or interest |
| - Team Collaboration and Communication Shares tasks and tools - Communicates clearly - Listens actively - Offers help when needed | 1.5 | Collaborates actively, supports teammates, and communicates well | Generally, works well with others, communicates adequately | Occasionally cooperative; needs encouragement | Disruptive or disengaged from team effort |
| - Safety and Work Cleanliness Uses PPE properly - Handles tools safely - Maintains clean and organized area - Follows safety protocols | 1.5 | Strictly follows all safety and cleanliness standards | Usually safe and tidy, with few lapses | Sometimes forgets safety or leaves area messy | Unsafe work habits or messy station |
| Total Score | 6 |  |  |  |  |
|  | 10 |  |  |  |  |

BET 22 Lesson 5 Rewinding and Repair of Split-Phase and Capacitor Type Motors

SINGLE-PHASE ELECTRIC MOTOR REWINDING

Individual Task Participation Rubric

| Criteria | Score | Excellent (2.5 pts) | Good (2 pts) | Satisfactory (1.5 pts) | Needs Improvement (1 pt) |
| --- | --- | --- | --- | --- | --- |
| - Initiative and Task Ownership Voluntarily starts assigned work - Prepares tools and materials - Organizes own workflow - Takes responsibility for tasks | 2.5 | Always initiates and completes tasks independently and on time | Often starts tasks with minimal prompting | Sometimes needs guidance and reminders | Rarely initiates or avoids responsibility |
| - Technical Involvement Winding coils - Stripping and cleaning wires - Wiring and connections - Assembly and mechanical fitting - Testing with measuring tools | 2.5 | Performs all technical tasks with high skill, accuracy, and safety | Completes most technical tasks with minor errors | Contributes with assistance; moderate errors observed | Limited participation; lacks skill or interest |
| - Team Collaboration and Communication Shares tasks and tools - Communicates clearly - Listens actively - Offers help when needed | 2.5 | Collaborates actively, supports teammates, and communicates well | Generally, works well with others, communicates adequately | Occasionally cooperative; needs encouragement | Disruptive or disengaged from team effort |
| - Safety and Work Cleanliness Uses PPE properly - Handles tools safely - Maintains clean and organized area - Follows safety protocols | 1.5 | Strictly follows all safety and cleanliness standards | Usually safe and tidy, with few lapses | Sometimes forgets safety or leaves area messy | Unsafe work habits or messy station |
| Total Score | 9 |  |  |  |  |
|  | 10 |  |  |  |  |